RL-3812-ICRGB

Specifications

| Picture | Dimension | Parameters |  |
| --- | --- | --- | --- |
|  |  | Input voltage | DC 24V |
|  |  | Lamp power | 2.88W |
|  |  | Chip | UCS 2903 |
|  |  | LED chip | SMD 3535 |
|  |  | LEDs number | 12 |
|  |  | Color | RGB programmable |
|  |  | IP rating | IP65 |
|  |  | Beam angle | ≥140° |
|  |  | Lifespan | ≥50000h |

Features

- This product is made up of PCB. Milky/Transparent plastic cover and base, a couple of 3-pins male and female connectors. As well as a series of gluing process to ensure good water and UV resistance.

- It is a RGB programmable product, can achieve many different light effects by using a LED controller such as K-1000c.

- The features of this product are high brightness, low power dissipation, long lifespan, low voltage decline and excellent thermal management and lumen maintenance.

- Customized requests are available, for example: logo, waterproof connectors, transparent or milky cover, and length of cables.

- This product has CE and RoHS certifications.

Applications

- Amusement Park Rides Light

Ferris wheel, Carousels, Pirate ship, Pendulum, Bumper car, Tower and so on.

- Advertisement Outdoor Signboard Backlight

Logo, Signage, Channel letter, Signs, Exposed luminous characters, Billboard and so on.

- Stage Decoration Light

KTV, Disco, Bar, Night club and other stage light.

- Commercial Light

Hotel, Shopping mall, Theater, Square and so on.

- Kids Rides Decoration

Castles, Robot, Mini carousels, Car, Mini train etc.

Packing & Weight

| Weight/100pcs | Pieces/bag | Pieces/carton | G.W/carton | Carton size |
| --- | --- | --- | --- | --- |
| 1.4kg | 100pcs | 480pcs | 16kg | 46.5cm*46cm*28cm |

Connection

Notes:

- Male connector (with pins) is signal input port, female connector (without pins) is signal output port.

- We suggest that every 20pcs lights connect to the power supply in parallel to avoid voltage decline.

- One controller can only support up to 2000 pixels, more than 2000 pixels, you need to cascade with multiple controllers.